Metal Box

Polymer Box

Panel Protector / Hardwire / M200/120/208 3Y 2nd Edition

Specifications - Operating:				Dry Contacts

8 3/4

8

Maximum Operating Voltage:				208 Volts AC

Typical Leakage Current:					<5µ amps

Operation Temperature:					- 40 to +85°C

Connectors:							Hardwired

Lines Protected:						L-L, L-G, N-G

Installation Configuration:						Parallel

4

Specifications - Electronic:

6 1/4

Maximum Surge Current (8x20µs):				600,000A total, 200KA per Phase

Maximum Surge Voltage (1.2x50µs):			10,000 Volts

Capacitance:							<2,000 pf

Clamping Voltage:						250 Volts RMS

Clamping Response Time:					<5 nanoseconds

Current:							Non-Load Bearing

Voltage:							Voltage Sensitive

Design:							(3) Thermally Fused Hybrid

Operation Indicators & Audible Alarm			(4) Light Emitting Diode (LED)

Power Dissipation (8x20µs):					102,000,000 Volts Amps

Energy Dissipation:						7600 Joules

UL 1449:							1449 Second Edition

UL and CUL Approved:					United States / Canada

Dimension:							Depth 3 1/2"

Length 8 3/4"(including tabs)

Width 6 1/4"

Weight:							Approximately 7.5 lbs Nema 12 Box

### Pass Voltage Tested to ANSI/IEEE B3 Impulse:	(L-L)800VoltsPeak(L-G)400V PK (L-N)400V Pk(N-G)400V Pk

Stormin surge suppressors have a Ten Year Product Warranty.  Please see warranty statement for complete product warranty information.

Page11

Click here for Index                       Click here for Page10                             Click here for Page 12

Panel Protector / Hardwire / PTS 208 3Y H / 2nd Edition

Metal Box

Polymer Box

# Specifications - Operating:

Maximum Operating Voltage:				208 Volts AC

Typical Leakage Current:					<5µ amps

Operation Temperature:					- 40 to +85°C

Connectors:							Hardwired

Lines Protected:						L-L, L-N, N-G, L-G

### Installation Configuration:					Parallel

# Specifications - Electronic:

Maximum Surge Current (8x20µs):				300,000 Amps Total

Maximum Surge Voltage (1.2x50µs):			10,000 Volts

Capacitance:							<2,000 pf

Clamping Voltage:						250 Volts RMS

Clamping Response Time:					<5 nanoseconds

Current:							Non-Load Bearing

Voltage:							Voltage Sensitive

Design:							Thermally Fused Hybrid

Operation Indicators:						Light Emitting Diode (LED)

Power Dissipation (8x20µs):					39,000,000 Volts Amps

Energy Dissipation:						2850 Joules

UL 1449:							1449 Second Edition

UL Recognized Component:					File E121637

UL and CUL Approved:					United States / Canada

Dimension:							Depth 2.500"

Width 4.750"

Length 5.800" (including tabs)

Weight:							Approximately 2 lbs.

### Pass Voltage Tested to ANSI/IEEE B3 Impulse:		UL’s Best 330 V Peak Rating

E-Clips surge suppressors have a Ten Year Product Warranty.  Please see warranty statement for complete product warranty information.

Click for Index			Click Here for Page 10				Click here for Page 12